#!/usr/bin/env python

# coding: utf-8

# In[24]:

import os

import s4l_v1.document as document

import s4l_v1.analysis as analysis

import s4l_v1.materials.database as database

import s4l_v1.model as model

import s4l_v1.simulation.emlf as emlf

import s4l_v1.units as units

from s4l_v1 import ReleaseVersion

from s4l_v1 import Unit

import s4l_v1 as s4l

from s4l_v1.data import TutorialDataDirectory

import numpy as np

import numpy

from pathlib import Path

import ModelingPostPro

import XPostProcessor as xp

import XCoreModeling as xcm

import time

def loadModel (dir_path=None):

filepath = os.path.join(dir_path, "nerve_model_electrode_noaxons.sab")

print(filepath)

#assert os.path.exists(filepath), "Invalid path: '%s'"%filepath

# set modeling unit to mm

model.SetLengthUnits( units.MilliMeter )  # not reflected in GUI!

print('Despite the display in the GUI, the model unit is', model.LengthUnits())

model.Import(filepath) #[0].Entities

def Create_Axon_Distribution(edge_length, minimum_edge_length):

ents = s4l.model.AllEntities()

axon_ent = s4l.model.CreateGroup("Axons_Folder")

point_ent = s4l.model.CreateGroup("Points_Folder")

options = xcm.MeshingOptions()

options.EdgeLength = edge_length  # 0.045# To change to change the density of points

options.MinEdgeLength = minimum_edge_length  # 0.013

ns = 1  # downsampling factor

cnt = 0

cn = 0

axonn = 0

for ent in ents["Fascicles_Folder"].Entities:  # type: ignore

# Gets the bounding box of the fascicle

entity_bounds = xcm.GetBoundingBox([ent])

L = 0.95 * (entity_bounds[1][2] - entity_bounds[0][2])

z0 = 0.5 * (entity_bounds[1][2] + entity_bounds[0][2])

# Creates a slice of the fascicle at given location

slice = xcm.CreatePlanarSlice(ent, xcm.Vec3(0, 0, 0), xcm.Vec3(0, 0, 1), 1)

xcm.CoverWireBody(slice)

slice.Name = "tempslice_" + ent.Name

name = ent.Name

slice = xcm.ConvertToTriangleMesh(slice)

slice.Name = name

# slice=xcm.ConvertToTriangleMesh(slice,max_edge_length=0.1) #target=0.1, min=0.001

xcm.RemeshTriangleMesh(slice, options)

# Creates a subfolder of axons and point for each fascicle

entfold = s4l.model.CreateGroup("S_Points_" + name)

splines = s4l.model.CreateGroup("S_Splines_" + name)

# Gets the surface of the fascicle and calculates the equivalent diameter

surf = xcm.MeasureArea([slice])

diam = 2 * np.sqrt(surf / np.pi)

# Creates a surface mesh on the fascicle cross section. The centers of

# the triangles are candidate location of the axons

faceter = s4l.analysis.core.ModelToGridFilter()

faceter.Entity = slice

faceter.MaximumEdgeLength = 0.3 * diam

faceter.Update(0)

target_grid = faceter.GetOutput(0)

assert target_grid is not None

# L=86

cm = 0

# Creates the axon trajectories

for i in range(0, target_grid.NumberOfCells, ns):

p = 1e3 * target_grid.GetCellCenter(i)

point = s4l.model.CreatePoint(p)

point.Name = "P_" + name + "_" + str(cm) + "_Random"

entfold.Add(point)

# z = np.random.uniform(zz_min,zz_max)

L1 = L * np.random.normal(1, 0.025)

x = p[0]

y = p[1]

z = p[2]

print(f"z {z} never used, only z0 {z0}")

a = s4l.model.CreateSpline(

[xcm.Vec3(x, y, z0 - 0.5 * L1), xcm.Vec3(x, y, z0 + 0.5 * L1)]

)

a.Name = name + "_Spline_" + str(cm) + "_Random"

splines.Add(a)

cm += 1

cn += 1

axonn += 1

cnt += 1

# Adds the axon trajectories and the point in the folders

axon_ent.Add(splines)

point_ent.Add(entfold)

# Delete the slice from the model

slice.Delete()

print("Total number of axons:", axonn)

# Creates Unique Folder for Fibers and Points

fold = s4l.model.CreateGroup("Functionalization")

fold.Add(axon_ent)

fold.Add(point_ent)

return fold

def DiscretizeAxonModel():

''' In this function, the axon trajectories are converted in electrophysiology axon models of myelinated and

unmyelinated axons. Unmyelinated Sundt fibers are assigned with a 1um fiber diameter, while myelinated "rat"

fibers are assigned to 7um diameters '''

entities = s4l.model.AllEntities()

axons_folder=entities['Axons_Folder'].Entities

sundt_axons = []

rat_axons = []

for folder in axons_folder:

if '6' in folder.Name:

# Sundt 1um

for axon in folder.Entities:

sundt_axons.append(axon)

else:

# Rat 7um neurons

for axon in folder.Entities:

rat_axons.append(axon)

sundt_props = model.SundtNeuronProperties()

sundt_props.AxonDiameter = 1.0

discretized_sundt = model.CreateAxonNeurons(sundt_axons, sundt_props )

rat_props = model.RatNeuronProperties()

rat_props.AxonDiameter = 7.0

discretized_rat = model.CreateAxonNeurons(rat_axons, rat_props )

def CreatesEMSimulation():

simulation = emlf.ElectroQsOhmicSimulation()

document.AllSimulations.Add( simulation )

# Mapping the components and entities

component__plane_x = simulation.AllComponents["Plane X+"]

component__plane_x = simulation.AllComponents["Plane X-"]

component__background = simulation.AllComponents["Background"]

component__plane_y = simulation.AllComponents["Plane Y+"]

component__plane_y = simulation.AllComponents["Plane Y-"]

component__plane_z = simulation.AllComponents["Plane Z+"]

component__plane_z = simulation.AllComponents["Plane Z-"]

component__overall_field = simulation.AllComponents["Overall Field"]

entity__blood2 = model.AllEntities()["Blood 2"]

entity__fascicle1 = model.AllEntities()["Fascicle 1"]

entity__fascicle3 = model.AllEntities()["Fascicle 3"]

entity__fascicle4 = model.AllEntities()["Fascicle 4"]

entity__fascicle2 = model.AllEntities()["Fascicle 2"]

entity__fascicle7 = model.AllEntities()["Fascicle 7"]

entity__electrode_1 = model.AllEntities()["Electrode_1"]

entity__connective_2b = model.AllEntities()["Connective_2b"]

entity__silicone = model.AllEntities()["Silicone"]

entity__blood3 = model.AllEntities()["Blood 3"]

entity__fascicle5 = model.AllEntities()["Fascicle 5"]

entity__connective_2a = model.AllEntities()["Connective_2a"]

entity__saline = model.AllEntities()["Saline"]

entity__connective = model.AllEntities()["Connective"]

entity__nerve = model.AllEntities()["Nerve"]

entity__interstitial = model.AllEntities()["Interstitial"]

entity__fascicle6 = model.AllEntities()["Fascicle 6"]

entity__blood1 = model.AllEntities()["Blood 1"]

entity__electrode_2 = model.AllEntities()["Electrode_2"]

# Adding a new MaterialSettings

material_settings = emlf.MaterialSettings()

components = [entity__silicone]

material_settings.Name = "Silicone"

simulation.Add(material_settings, components)

material_settings = emlf.MaterialSettings()

components = [entity__interstitial]

mat = database["IT'IS LF 4.2"]["Fat"]

if mat is not None:

simulation.LinkMaterialWithDatabase(material_settings, mat)

else:

# Fallback if material is not found

material_settings.Name = "Fat"

material_settings.MassDensity = 911.0, Unit("kg/m^3")

material_settings.ElectricProps.Conductivity = 0.07762127305463426, Unit("S/m")

material_settings.ElectricProps.RelativePermittivity = 19286.645785175333

simulation.Add(material_settings, components)

material_settings = emlf.MaterialSettings()

components = [entity__saline]

material_settings.Name = "Saline"

material_settings.ElectricProps.Conductivity = 2.0, Unit("S/m")

simulation.Add(material_settings, components)

material_settings = emlf.MaterialSettings()

components = [entity__blood1, entity__blood2, entity__blood3]

mat = database["IT'IS LF 4.2"]["Blood"]

if mat is not None:

simulation.LinkMaterialWithDatabase(material_settings, mat)

else:

# Fallback if material is not found

material_settings.Name = "Blood"

material_settings.MassDensity = 1049.75, Unit("kg/m^3")

material_settings.ElectricProps.Conductivity = 0.6624597361833767, Unit("S/m")

material_settings.ElectricProps.RelativePermittivity = 5258.608390020375

simulation.Add(material_settings, components)

material_settings = emlf.MaterialSettings()

components = [entity__nerve]

mat = database["IT'IS LF 4.2"]["Nerve"]

if mat is not None:

simulation.LinkMaterialWithDatabase(material_settings, mat)

else:

# Fallback if material is not found

material_settings.Name = "Nerve"

material_settings.MassDensity = 1075.0, Unit("kg/m^3")

material_settings.ElectricProps.Conductivity = 0.3479543931346832, Unit("S/m")

material_settings.ElectricProps.RelativePermittivity = 69911.4914652573

simulation.Add(material_settings, components)

material_settings = emlf.MaterialSettings()

components = [entity__fascicle1, entity__fascicle2, entity__fascicle3, entity__fascicle4, entity__fascicle5, entity__fascicle6, entity__fascicle7]

material_settings.Name = "Fascicles"

material_settings.ElectricProps.ConductivityAnisotropic = True

material_settings.ElectricProps.ConductivityDiagonalElements = numpy.array([0.086, 0.086, 0.571]), Unit("S/m")

simulation.Add(material_settings, components)

material_settings = emlf.MaterialSettings()

components = [entity__connective, entity__connective_2a, entity__connective_2b]

mat = database["IT'IS LF 4.2"]["Connective Tissue"]

if mat is not None:

simulation.LinkMaterialWithDatabase(material_settings, mat)

else:

# Fallback if material is not found

material_settings.Name = "Connective Tissue"

material_settings.MassDensity = 1026.5, Unit("kg/m^3")

material_settings.ElectricProps.Conductivity = 0.07919585744745661, Unit("S/m")

material_settings.ElectricProps.RelativePermittivity = 302705.19018286216

simulation.Add(material_settings, components)

# Adds the boundary conditions

boundary_settings = [x for x in simulation.AllSettings if isinstance(x, emlf.BoundarySettings) and x.Name == "Boundary Settings"][0]

components = [component__plane_x, component__plane_x, component__plane_y, component__plane_y, component__plane_z, component__plane_z]

simulation.Add(boundary_settings, components)

boundary_settings.BoundaryType = boundary_settings.BoundaryType.enum.Flux

boundary_settings = emlf.BoundarySettings()

components = [entity__electrode_1]

boundary_settings.Name = "Boundary Settings 1"

boundary_settings.DirichletValue = -0.5, units.Volts

simulation.Add(boundary_settings, components)

boundary_settings = emlf.BoundarySettings()

components = [entity__electrode_2]

boundary_settings.Name = "Boundary Settings 2"

boundary_settings.DirichletValue = 0.5, units.Volts

simulation.Add(boundary_settings, components)

# Editing GlobalGridSettings "Grid (Empty)

global_grid_settings = simulation.GlobalGridSettings

global_grid_settings.DiscretizationMode = global_grid_settings.DiscretizationMode.enum.Manual

global_grid_settings.MaxStep = numpy.array([10.0, 10.0, 10.0]), units.MilliMeters

global_grid_settings.Resolution = numpy.array([0.625, 0.625, 0.625]), units.MilliMeters

global_grid_settings.PaddingMode = global_grid_settings.PaddingMode.enum.Manual

global_grid_settings.BottomPadding = numpy.array([1.0, 1.0, 1.0]), units.MilliMeters

global_grid_settings.TopPadding = numpy.array([1.0, 1.0, 1.0]), units.MilliMeters

manual_grid_settings = simulation.AddManualGridSettings([entity__blood1, entity__blood2, entity__blood3, entity__connective, entity__connective_2a, entity__connective_2b, entity__electrode_1, entity__electrode_2, entity__fascicle1, entity__fascicle2, entity__fascicle3, entity__fascicle4, entity__fascicle5, entity__fascicle6, entity__fascicle7, entity__interstitial, entity__nerve, entity__saline, entity__silicone])

manual_grid_settings.MaxStep = numpy.array([0.025, 0.025, 0.06]), units.MilliMeters

manual_grid_settings.Priority = 0.0

# Adding a new ManualVoxelerSettings

automatic_voxeler_settings = [x for x in simulation.AllSettings if isinstance(x, emlf.AutomaticVoxelerSettings) and x.Name == "Automatic Voxeler Settings"][0]

automatic_voxeler_settings.Priority = 2

manual_voxeler_settings = emlf.ManualVoxelerSettings()

components = [entity__fascicle1, entity__fascicle2, entity__fascicle3, entity__fascicle4, entity__fascicle5, entity__fascicle6, entity__fascicle7]

manual_voxeler_settings.Name = "Fascicles"

manual_voxeler_settings.Priority = 8

simulation.Add(manual_voxeler_settings, components)

manual_voxeler_settings = emlf.ManualVoxelerSettings()

components = [entity__silicone]

manual_voxeler_settings.Name = "Silicone"

manual_voxeler_settings.Priority = 9

simulation.Add(manual_voxeler_settings, components)

manual_voxeler_settings = emlf.ManualVoxelerSettings()

components = [entity__electrode_1, entity__electrode_2]

manual_voxeler_settings.Name = "Electrodes"

manual_voxeler_settings.Priority = 10

simulation.Add(manual_voxeler_settings, components)

manual_voxeler_settings = emlf.ManualVoxelerSettings()

components = [entity__nerve]

manual_voxeler_settings.Name = "Nerve"

manual_voxeler_settings.Priority = 4

simulation.Add(manual_voxeler_settings, components)

manual_voxeler_settings = emlf.ManualVoxelerSettings()

components = [entity__interstitial]

manual_voxeler_settings.Name = "Interstitial"

manual_voxeler_settings.Priority = 5

simulation.Add(manual_voxeler_settings, components)

manual_voxeler_settings = emlf.ManualVoxelerSettings()

components = [entity__blood1, entity__blood2, entity__blood3]

manual_voxeler_settings.Name = "Blood"

manual_voxeler_settings.Priority = 6

simulation.Add(manual_voxeler_settings, components)

manual_voxeler_settings = emlf.ManualVoxelerSettings()

components = [entity__saline]

manual_voxeler_settings.Name = "Saline"

simulation.Add(manual_voxeler_settings, components)

manual_voxeler_settings = emlf.ManualVoxelerSettings()

components = [entity__connective, entity__connective_2a, entity__connective_2b]

manual_voxeler_settings.Name = "Connective"

manual_voxeler_settings.Priority = 7

simulation.Add(manual_voxeler_settings, components)

# Editing SolverSettings "Solver

solver_settings = simulation.SolverSettings

solver_settings.PredefinedTolerances = solver_settings.PredefinedTolerances.enum.High

# Update the materials and the grid

simulation.UpdateAllMaterials()

simulation.UpdateGrid()

# Create the voxels

simulation.CreateVoxels()

return simulation

def CreateNeuronSimulation(strategy):

import s4l_v1.simulation.neuron as neuron

# Create a Neuron simulation

sim = s4l.simulation.neuron.Simulation()

sim.Name = "Neuronal Simulation Titration_"+strategy

# Add the simulation to the UI

document.AllSimulations.Add(sim)

# Setup Settings

setup_settings = sim.SetupSettings

setup_settings.PerformTitration = True

if strategy=='NEURON':

sim.SetupSettings.TitrationStrategy=0

elif strategy=='Estimator':

sim.SetupSettings.TitrationStrategy=1

# Neuron Settings

entities = model.AllEntities()

axons = [ent for ent in entities if '[Rat Neuron 7.00um]' in ent.Name or '[Unmyelinated Sundt Neuron 1.00um]' in ent.Name]

neuron_settings = neuron.AutomaticAxonNeuronSettings()

sim.Add(neuron_settings, axons)

# Source Settings

# get incident field sensor component from lf simulation

incident_sim = document.AllSimulations["LF"]

assert incident_sim is not None, "Electromagnetic simulation to be used in the Neuron simulation could not be found."

source_settings = sim.AddSource(incident_sim, "Overall Field")

source_settings.PulseType = source_settings.PulseType.enum.Bipolar

source_settings.InitialTime = 0.1

source_settings.AmplitudeP1 = 1.0

source_settings.DurationP1 = 0.5

source_settings.Interval = 0.1

# Sensor Settings

# Fascicle 1 spline 1

axon1 = [axon for axon in sim.GetNeuronEntities() if axon.Name == "Fascicle 1_Spline_0_Random [Rat Neuron 7.00um]"][0]

point_sensor_settings = sim.AddPointSensor(axon1)

point_sensor_settings.SectionName = "node[2]"

point_sensor_settings.RelativePosition = 0.5

point_sensor_settings = sim.AddPointSensor(axon1)

point_sensor_settings.SectionName = "node[8]"

point_sensor_settings.RelativePosition = 0.5

point_sensor_settings = sim.AddPointSensor(axon1)

point_sensor_settings.SectionName = "node[17]"

point_sensor_settings.RelativePosition = 0.5

# Fascicle 6 spline 1

axon2 = [axon for axon in sim.GetNeuronEntities() if axon.Name == "Fascicle 6_Spline_0_Random [Unmyelinated Sundt Neuron 1.00um]"][0]

point_sensor_settings = sim.AddPointSensor(axon2)

point_sensor_settings.SectionName = "section[70]"

point_sensor_settings.RelativePosition = 0.5

point_sensor_settings = sim.AddPointSensor(axon2)

point_sensor_settings.SectionName = "section[169]"

point_sensor_settings.RelativePosition = 0.5

point_sensor_settings = sim.AddPointSensor(axon2)

point_sensor_settings.SectionName = "section[286]"

point_sensor_settings.RelativePosition = 0.5

# solver settings

solver_settings = sim.SolverSettings

solver_settings.Duration = 2.5

solver_settings.NumberOfThreads = 6

solver_settings.TimeStep = 0.00025

# Rat membrane dynamics are very fast, and thus need a smaller timestep

return sim

def AnalyzeSimulation(sim, plot_v = False):

# Adding a new SimulationExtractor

#assert sim.HasResults(), f"Provided simulation {sim.Name} does not yet have results. Run it to be able to analyze it"

# Adding a new SimulationExtractor

simulation_extractor = sim.Results()

# Adding a new TitrationEvaluator

inputs = [simulation_extractor["Titration Sensor"]["Titration"]]

titration_evaluator = analysis.neuron_evaluators.TitrationEvaluator(inputs=inputs)

titration_evaluator.UpdateAttributes()

titration_evaluator.Update(0)

document.AllAlgorithms.Add(titration_evaluator)

# Extracting the Titration information for further analysis

tf=titration_evaluator.TitrationFactor      # titration factor

tl=titration_evaluator.LocationOfFirstSpike # section of the first spike

ts=titration_evaluator.TimeOfFirstSpike     # latency, in [ms]

neuron_names=titration_evaluator.NeuronName        # name of the neuron spiking

thresholds=np.array(tf)

if plot_v:

# Adding a new SensorExtractor

sensor_extractor = simulation_extractor["Fascicle 1_Spline_0_Random [Rat Neuron 7.00um]  (Functionalization/Axons_Folder/S_Splines_Fascicle 1)@node[17]@0.50"]

document.AllAlgorithms.Add(sensor_extractor)

# Adding a new SensorExtractor

sensor_extractor_2 = simulation_extractor["Fascicle 1_Spline_0_Random [Rat Neuron 7.00um]  (Functionalization/Axons_Folder/S_Splines_Fascicle 1)@node[2]@0.50"]

document.AllAlgorithms.Add(sensor_extractor_2)

# Adding a new SensorExtractor

sensor_extractor_3 = simulation_extractor["Fascicle 1_Spline_0_Random [Rat Neuron 7.00um]  (Functionalization/Axons_Folder/S_Splines_Fascicle 1)@node[8]@0.50"]

document.AllAlgorithms.Add(sensor_extractor_3)

# Adding a new SensorExtractor

sensor_extractor_4 = simulation_extractor["Fascicle 6_Spline_0_Random [Unmyelinated Sundt Neuron 1.00um]  (Functionalization/Axons_Folder/S_Splines_Fascicle 6)@section[169]@0.50"]

document.AllAlgorithms.Add(sensor_extractor_4)

# Adding a new SensorExtractor

sensor_extractor_5 = simulation_extractor["Fascicle 6_Spline_0_Random [Unmyelinated Sundt Neuron 1.00um]  (Functionalization/Axons_Folder/S_Splines_Fascicle 6)@section[286]@0.50"]

document.AllAlgorithms.Add(sensor_extractor_5)

# Adding a new SensorExtractor

sensor_extractor_6 = simulation_extractor["Fascicle 6_Spline_0_Random [Unmyelinated Sundt Neuron 1.00um]  (Functionalization/Axons_Folder/S_Splines_Fascicle 6)@section[70]@0.50"]

document.AllAlgorithms.Add(sensor_extractor_6)

# Adding a new PlotViewer

inputs = [sensor_extractor.Outputs["v"], sensor_extractor_2.Outputs["v"], sensor_extractor_3.Outputs["v"], sensor_extractor_4.Outputs["v"], sensor_extractor_5.Outputs["v"], sensor_extractor_6.Outputs["v"]]

plot_viewer = analysis.viewers.PlotViewer(inputs=inputs)

plot_viewer.UpdateAttributes()

document.AllAlgorithms.Add(plot_viewer)

return [neuron_names,thresholds]

def Calculates_Recruitment_Curve(simulation,N):

""" Calculates the recruitment percentage of axons/neurons for a range of threshold currents.

Notes: users should use this script with care and adapt it at occurrence for the specific

investigated system. The provide calculations assumes that each axon/neuron has the same

weight with respect to recruitment. There can be cases where this assumption is not valid.

Parameters:

N (int): Number of points in the log-spaced current range.

Returns:

numpy.ndarray: Recruitment percentages.

"""

assert simulation.HasResults()==1, "Neuronal Simulation Should Have Results"

simulation_extractor = simulation.Results()

# Adding a new SensorExtractor

sensor_extractor = simulation_extractor["Titration Sensor"]

sensor_extractor.Update()

document.AllAlgorithms.Add(sensor_extractor)

# Adding a new TitrationEvaluator

inputs = [sensor_extractor.Outputs["Titration"]]

titration_evaluator = analysis.neuron_evaluators.TitrationEvaluator(inputs=inputs)

titration_evaluator.UpdateAttributes()

titration_evaluator.Update()

document.AllAlgorithms.Add(titration_evaluator)

tf=titration_evaluator.TitrationFactor      # titration factor

tl=titration_evaluator.LocationOfFirstSpike # section of the first spike

ts=titration_evaluator.TimeOfFirstSpike     # latency, in [ms]

nname=titration_evaluator.NeuronName        # name of the neuron spiking

# Calculates the recruitment index (see note)

Naxons = len(tf)

currents = np.logspace(np.log10(np.min(tf)), np.log10(np.max(tf)), N)

recruitment = np.array([np.count_nonzero(tf <= c) / Naxons * 100 for c in currents])

return [currents,recruitment]

import matplotlib.pyplot as plt

import numpy as np

def PlotXY(x, y, xname, xunit, yname, yunit, title):

"""

Given two numpy vectors x and y, the function creates a 2D matplotlib plot including axis labels and a title.

Parameters:

x (np.ndarray): x-axis data

y (np.ndarray): y-axis data

xname (str): Label for x-axis quantity

xunit (str): Unit of x-axis

yname (str): Label for y-axis quantity

yunit (str): Unit of y-axis

title (str): Plot title

"""

# Ensure x and y are numpy arrays

x = np.asarray(x)

y = np.asarray(y)

# Create the plot

plt.figure(figsize=(8, 5))

plt.plot(x, y, label=f'{yname} vs {xname}', color='cadetblue')  # You can customize the color

plt.xlabel(f'{xname} ({xunit})')

plt.ylabel(f'{yname} ({yunit})')

plt.title(title)

plt.grid(True)

plt.legend()

plt.tight_layout()

plt.show()

def Calculates_Selectivity_Indices(simulation):

'''Given a electrophysiology simulation and a rule to assign neurons(axons) to groups (e.g. fascicles),

the script calculates the selectivity indices.'''

# Adding a new SimulationExtractor

simulation_extractor_2 = simulation.Results()

# Adding a new SensorExtractor

#print('Sensors ', simulation_extractor_2.keys())

sensor_extractor = simulation_extractor_2["Action Potential Sensor"]

document.AllAlgorithms.Add(sensor_extractor)

# Adding a new SelectivityEvaluator

inputs = [sensor_extractor.Outputs["Action Potential"]]

selectivity_evaluator = s4l.analysis.neuron_evaluators.SelectivityEvaluator(inputs=inputs)

selectivity_evaluator.UpdateAttributes()

document.AllAlgorithms.Add(selectivity_evaluator)

# Adds the algorithm to the GUI

s4l.document.AllAlgorithms.Add(selectivity_evaluator)

# Gets the default assignment of axons to groups

#my_assignment = selectivity_evaluator.raw.Assignment

# Gets the axons. In their name the axon diameter is specified

axons=[k for k in selectivity_evaluator.Assignment.keys()]

#axons=[a.Name for a in simulation.GetNeuronEntities()]

# In this example, axon names include the fascicle ID. Now I assign each axon to its group/fascicle

for axon in axons:

group=int(axon.split("_")[0].split(" ")[1])

#my_assignment[axon]=group

selectivity_evaluator.SetNeuronAssignment(axon, group)

selectivity_evaluator.UpdateAttributes()

selectivity_evaluator.Update()

#print (selectivity_evaluator.GetNeuronAssignment(axon))

#evaluator.SetNeuronAssignment('Fascicle 1_Spline_0 [SENN Neuron 5.00um]  (Axons_Folder/Splines_Fascicle 1)',20)

#print(selectivity_evaluator.Assignment)

# Provide the assignment to the algorithm

#evaluator.raw.Assignment = my_assignment

selectivity_evaluator.UpdateAttributes()

selectivity_evaluator.Update()

# Calculates the selectivity indices

evalu=selectivity_evaluator

results=evalu.SelectivityMap()

groups=np.array(list(results.keys()))

selectivity_indices=np.array(list(results.values()))

return [groups,selectivity_indices]

def Calculates_Selectivity_Curve(simulation, currents):

selectivities=[]

# Creates Recruitment Curves

N=10

# Calculate Current-Dependent Selectivity Indices

for current in currents:

simulation.ClearResults()

# Remove the titration option

simulation.SetupSettings.PerformTitration=False

sets=simulation.AllSettings

ss=sets['Source Settings']

ss.AmplitudeP1=current

simulation.RunSimulation()

if simulation.HasResults():

print('Computing Selectivity for current ', current)

[groups,selectivity_index]=Calculates_Selectivity_Indices(simulation)

selectivities.append(selectivity_index)

selectivities=np.array(selectivities)

legend=[str(g) for g in groups]

return [selectivities,legend]

import matplotlib.pyplot as plt

import numpy as np

def Plot_MultiXY(x,y,xname,xunit,yname,yunit,title,labels):

# Make sure y is transposed correctly if needed

y = np.transpose(y)

# Plotting

plt.figure(figsize=(10, 6))

for i in range(y.shape[0]):

plt.plot(x, y[i, :], label=labels[i])

# Set axis labels with units

plt.xlabel(f"{xname} ({xunit})")

plt.ylabel(f"{yname} ({yunit})")

# Set title and legend

plt.title(f"{title}")

plt.legend()

# Show plot

plt.grid(True)

plt.tight_layout()

plt.show()

document.New()

#Loads the CAD model of the nerve and electrodes

loadModel(r"C:\Users\Public\Documents\ZMT\Sim4Life\9.0\Documentation\Tutorials\Tutorial Projects\CAD_files")

# Creates a list of Entities

ents=s4l.model.AllEntities()

# #### **Extracts the fascicles as Entities - Required for Neuro-Functionalization**

folder=s4l.model.CreateGroup('Fascicles_Folder') # Folder where all Fascicle Entities are Grouped

for e in ents:

if hasattr(e, 'Name') and 'Fascicle ' in e.Name:

print(f"Adding: {e.Name}")

folder.Add(e)

Create_Axon_Distribution(0.25, 0.05)

# Creates the EM simulation

em_simulation=CreatesEMSimulation()

# Runs the EM simulation

em_simulation.RunSimulation(wait=True)

# Fields Normalized to 1mA

simulation_extractor = em_simulation.Results()

# Adding a new EmSensorExtractor

em_sensor_extractor = simulation_extractor["Overall Field"]

em_sensor_extractor.FrequencySettings.ExtractedFrequency = u"All"

em_sensor_extractor.Normalization.Normalize = True

em_sensor_extractor.Normalization.NewReferenceValue = 0.001 # Fields normalized to 1mA input current

document.AllAlgorithms.Add(em_sensor_extractor)

inputs = [em_sensor_extractor.Outputs["EM E(x,y,z,f0)"]]

slice_field_viewer = analysis.viewers.SliceFieldViewer(inputs=inputs)

slice_field_viewer.Data.Mode = slice_field_viewer.Data.Mode.enum.QuantityComplexModulus

slice_field_viewer.Data.Component = slice_field_viewer.Data.Component.enum.ComponentsAll

slice_field_viewer.Slice.SliceOrigin = np.array([0.0008823550888337195, 0.0006480618030764163, 0.009349999949336052])

slice_field_viewer.Slice.Normal = np.array([0.0, 1.0, 0.0])

slice_field_viewer.UpdateAttributes()

slice_field_viewer.Update()

document.AllAlgorithms.Add(slice_field_viewer)

# Discretize Axon Fibers

DiscretizeAxonModel()

# Creates and runs the Neuronal Simulation (Complete Electrophysiological Simulation, NEURON-based)

simNeuro = CreateNeuronSimulation(strategy='Neuron')

# Runs the Neuronal Simulation

t0=time.time()

simNeuro.RunSimulation()

tn=time.time()-t0

print ('Execution Titration - NEURON approach - duration ', tn, ' seconds')

# Creates and runs the Neuronal Simulation (Estimator, GAF-baseed)

simEstimator = CreateNeuronSimulation(strategy='Estimator')

# Runs the Neuronal Simulation

t0=time.time()

simEstimator.RunSimulation()

tn=time.time()-t0

print ('Execution Titration - Estimator approach - duration ', tn, ' seconds')

# Reporting acceleration using the Estimator strategy over the Neuron strategy

#print('Estimator vs NEURON execution time ratio ', te/tn, 'Acceleration: ',tn/te,' times')

[neuron_namesN, thresholdsN] = AnalyzeSimulation(simNeuro, plot_v = True)

[neuron_namesE, thresholdsE] = AnalyzeSimulation(simEstimator)

# Plot titration factor estimation per fiber, both from NEURON and GAF results

fiber_indexes = np.arange(1, len(thresholdsN)+1).astype(float).tolist() # Create an array of fiber indices

legend = ['NEURON', 'Estimator'] # Create a legend for the plot

thresholds_all = np.vstack((thresholdsN, thresholdsE)).T #stack 2 arrays into a 2D array and transposes it

Plot_MultiXY(fiber_indexes,thresholds_all,"Fiber index","","Titration factor","","Titration factor prediction",legend)

import matplotlib.pyplot as plt

plt.figure()

is_rat = np.array([True if '[Rat Neuron 7.00um]' in axon else False for axon in neuron_namesN])

plt.scatter(thresholdsN[is_rat], thresholdsE[is_rat], c="b", label="Rat")

plt.scatter(thresholdsN[~is_rat], thresholdsE[~is_rat], c="r", label="Sundt")

plt.xscale("log")

plt.yscale("log")

plt.ylabel('Threshold Current [mA] - Estimator')

plt.xlabel('Threshold Current [mA] - Neuron')

m = np.min(list(thresholdsN) + list(thresholdsE))

M = np.max(list(thresholdsN) + list(thresholdsE))

plt.plot([m, M], [m, M], "k--", alpha=0.5)

plt.title('Comparison Threshold Estimator vs Neuron')

plt.grid(True)

ax = plt.gca()

ax.set_aspect('equal', 'box')

plt.xlim(m*0.8, M*1.2)

plt.ylim(m*0.8, M*1.2)

plt.legend()

#Compute the recruitment curve

[currents,recruitment] = Calculates_Recruitment_Curve(simNeuro,10)

# Plots the recruitment curve

PlotXY(currents,recruitment,'Threshold','mA','Recruitment Index','%','')

[selectivities, legend] = Calculates_Selectivity_Curve(simNeuro, currents)

# Plot Selectivity Curves

labels = [f'Fascicle {int(x)+1}' for x in legend]

#Plot_MultiXY(currents,selectivities,"Current","mA","Selectivity Index","","Selectivity Plot",labels)

Plot_MultiXY(currents,selectivities,"Current","mA","Selectivity Index","","Selectivity Plot",labels)