from collections import defaultdict

from pathlib import Path

import matplotlib.pyplot as plt

import numpy as np

import s4l_v1.analysis as analysis

import s4l_v1.document as document

import s4l_v1.materials.database as database

import s4l_v1.model as model

import s4l_v1.units as units

import XCoreModeling as xcm

import XPostProcessor as xp

from s4l_v1.data import TutorialDataDirectory

from s4l_v1.model import Vec3

from s4l_v1.renderer import SetViewDirection, ShowScreenCapture, ZoomToEntity

from s4l_v1.simulation import emlf, neuron

# parameters

TRUNCATE_DOMAIN = True

voltage_difference = 1.0  # V

#project_name = "eti_simple.smash"

import s4l_v1.ui  # noqa: E402

s4l_v1.ui.ChangeCurrentMode("Analysis")

# Extract EM results

em_simulation = document.AllSimulations["LF"]

em_simulation_extractor = em_simulation.Results()

em_sensor_extractor = em_simulation_extractor["Overall Field"]

em_sensor_extractor.UpdateAttributes()

# Compute currents

inputs = [

em_sensor_extractor.Outputs["EM Potential(x,y,z,f0)"],

em_sensor_extractor.Outputs["J(x,y,z,f0)"],

]

current_extractor = analysis.extractors.CurrentExtractor(inputs=inputs)

current_extractor.UpdateAttributes()

document.AllAlgorithms.Add(current_extractor)

current_extractor.Update()

c = current_extractor.Outputs["Total Flux(J(x,y,z,f0))"]

total_unnormalized_current = np.real(c.Data.GetComponent(0)[0])  # in A

model_impedance = voltage_difference / total_unnormalized_current  # in Ohms

print(

f"Un-normalized total current between the electrodes is: {1e3 * total_unnormalized_current:.3f} mA"

)

print(f"Bulk impedance is: {model_impedance:.1f} Ohms")

def create_data_producer(

x_array, data_array, x_name, x_unit, y_name, y_unit, name, iscomplex=True

):

if iscomplex:

data = xp.ComplexFloatXYData()

else:

data = xp.FloatXYData()

quantity = xp.Quantity()

quantity.Name = y_name

quantity.Unit = xp.Unit(y_unit)

data.Quantity = quantity

assert data.Allocate(len(data_array), [1])

data.Axis = x_array.astype(np.float64)

quantity = xp.Quantity()

quantity.Name = x_name

quantity.Unit = xp.Unit(x_unit)

data.AxisQuantity = quantity

if iscomplex:

data.SetComponent(0, data_array.astype(np.complex64))

else:

data.SetComponent(0, data_array.astype(np.float32))

# producer

producer = analysis.core.TrivialProducer()

producer.SetDataObject(data)

producer.Description = str(name)

document.AllAlgorithms.Add(producer)

return producer

# define short rectangular pulse

t_1 = 5e-3   # 脈衝開始時間，表示刺激從 5 ms 時開始

pulse_amplitude = 1 # 脈衝振幅

pulse_duration = 10e-3  # 脈衝持續時間

t_max = 30e-3 # 總模擬時間，定義整個時域訊號長度

Nt = 5001   # 時間取樣點數

dt = t_max / Nt # 時間步長，相鄰兩個時間點之間的時間間隔

t_array = np.linspace(0, t_max, Nt).astype(np.float32)

pulse_array = np.zeros_like(t_array)

pulse_array = np.where(

(t_array > t_1) & (t_array <= t_1 + pulse_duration),

pulse_amplitude,

pulse_array,

)

modulating_pulse_producer = create_data_producer(

t_array,

pulse_array,

"Time",

"s",

"Modulation(t)",

"",

"Modulating Pulse",

iscomplex=False,

)

fmax = 1 / dt

contact_impedance_model_producer = analysis.core.ContactImpedanceModelProducer(

inputs=[]

)

contact_impedance_model_producer.ModelType = (

contact_impedance_model_producer.ModelType.enum.ITIS

)

contact_impedance_model_producer.MaxFrequency = fmax, units.Hz

contact_impedance_model_producer.Steps = 5001

contact_impedance_model_producer.ItisSkinThickness = "Thin"

contact_impedance_model_producer.ItisElectrodeSize = 1.5, units.SquareCentiMeters

contact_impedance_model_producer.UpdateAttributes()

document.AllAlgorithms.Add(contact_impedance_model_producer)

contact_impedance_model_producer.Update()

Z_ETI = contact_impedance_model_producer.Outputs[0].Data.GetComponent(0)

f_array = contact_impedance_model_producer.Outputs[0].Data.Axis

fig, ax = plt.subplots()

ax.plot(f_array, np.real(Z_ETI), label="Re(Z)")

ax.plot(f_array, np.imag(Z_ETI), label="Im(Z)")

ax.set_xlabel("Frequency (Hz)")

ax.set_ylabel("Z(f) (Ohms)")

ax.set_title("Electrode/Skin Contact Impedance")

ax.legend()

fig.tight_layout()

plt.show()

f_array = np.concatenate([-f_array[-1:0:-1], f_array])

Z_ETI = np.concatenate([np.conj(Z_ETI[-1:0:-1]), Z_ETI])

Z_load = model_impedance

scaling_factor_array = Z_load / (2 * Z_ETI + Z_load)

scaling_factor_producer = create_data_producer(

f_array,

scaling_factor_array,

"Frequency",

"Hz",

"EM Z(f)",

"Ohm",

"Electrode/Skin/Bulk Impedance model",

iscomplex=True,

)

fig, ax = plt.subplots()

ax.plot(f_array, np.real(scaling_factor_array), label="Re")

ax.plot(f_array, np.imag(scaling_factor_array), label="Im")

ax.set_xlabel("Frequency (Hz)")

ax.set_ylabel("")

ax.set_title("Z_load / (Z_A + Z_B + Z_load)")

ax.legend()

fig.tight_layout()

plt.show()

# create and execute the algorithm

inputs = [modulating_pulse_producer.Outputs[0], scaling_factor_producer.Outputs[0]]

circuit_excitation = analysis.core.CircuitExcitation(inputs=inputs)

circuit_excitation.UpdateAttributes()

assert circuit_excitation.Update()

document.AllAlgorithms.Add(circuit_excitation)

t_array = modulating_pulse_producer.Outputs[0].Data.Axis

p_array = modulating_pulse_producer.Outputs[0].Data.GetComponent(0)

p2_array = circuit_excitation.Outputs[0].Data.GetComponent(0)

fig, ax = plt.subplots()

ax.plot(t_array, p_array, label="Original pulse")

ax.plot(t_array, p2_array, label="Pulse w/ Contact Impedance")

ax.set_xlabel("Time (s)")

ax.set_ylabel("Excitation signal")

ax.set_title("Excitation pulse")

ax.legend()

fig.tight_layout()

plt.show()

inputs = [em_sensor_extractor.Outputs[0], circuit_excitation.Outputs[0]]

modulated_field = analysis.core.ModulatedFieldEvaluator(inputs=inputs)

modulated_field.UpdateAttributes()

modulated_field.Name = "Modulated Efield"

document.AllAlgorithms.Add(modulated_field)