import numpy as np

import matplotlib.pyplot as plt

import ipywidgets as widgets

from IPython.display import display, clear_output

import os

style = {'description_width': 'initial'}

wave_type = widgets.Dropdown(

options=[('雙相方波 Biphasic Wave', 'biphasic'), ('單相方波 Square Wave', 'square'), ('弦波 Sine Wave', 'sine')],

value='biphasic', description='波形類型:', style=style

)

freq_in = widgets.FloatText(value=50.0, description='頻率 Frequency (Hz):', style=style)

pw_in = widgets.FloatText(value=8.0, description='單一脈寬 Pulse Width (ms):', style=style)

rep_in = widgets.IntText(value=10, description='重複次數 Repeat:', style=style)

path_in = widgets.Text(value=r'C:\Users\Patrick.yu\Desktop\Stim_Wave.modulation', description='儲存檔案:', style=style)

btn_preview = widgets.Button(description="更新預覽圖", button_style='info')

btn_save = widgets.Button(description="導出並自動換算為秒", button_style='success')

out = widgets.Output()

pw_container = widgets.VBox([pw_in])

def generate_wave_data(w_type, f, pw_ms, rep):

period_s = 1.0 / f

total_time_s = period_s * rep

if w_type == 'biphasic':

pw_s = pw_ms / 1000.0

times_s = []

values = []

curr_t = 0.0

if 2 * pw_s > period_s:

raise ValueError(f"錯誤：脈寬過大！在 {f}Hz 下，週期僅為 {period_s*1000}ms，無法容納兩個 {pw_ms}ms 的脈衝。")

for i in range(rep):

times_s.append(curr_t); values.append(1.0)

curr_t += pw_s

times_s.append(curr_t); values.append(1.0)

times_s.append(curr_t); values.append(-1.0)

curr_t += pw_s

times_s.append(curr_t); values.append(-1.0)

times_s.append(curr_t); values.append(0.0)

curr_t += (period_s - 2 * pw_s)

times_s.append(curr_t); values.append(0.0)

return np.array(times_s), np.array(values)

elif w_type == 'square':

pw_s = pw_ms / 1000.0

times_s = []

values = []

curr_t = 0.0

if pw_s > period_s:

raise ValueError(f"錯誤：脈寬過大！無法超過週期 {period_s*1000}ms。")

for i in range(rep):

times_s.append(curr_t); values.append(1.0)

curr_t += pw_s

times_s.append(curr_t); values.append(1.0)

times_s.append(curr_t); values.append(0.0)

curr_t += (period_s - pw_s)

times_s.append(curr_t); values.append(0.0)

return np.array(times_s), np.array(values)

else:

t_s = np.linspace(0, total_time_s, 200 * rep)

v = np.sin(2 * np.pi * f * t_s)

return t_s, v

def show_preview(b):

with out:

clear_output(wait=True)

try:

t_s, v = generate_wave_data(wave_type.value, freq_in.value, pw_in.value, rep_in.value)

plt.figure(figsize=(10, 3))

if wave_type.value in ['square', 'biphasic']:

plt.step(t_s * 1000, v, where='post', color='blue', lw=1.5)

else:

plt.plot(t_s * 1000, v, color='red', lw=1.5)

plt.title(f"Preview ({wave_type.value} | UI displays ms | File saves as s)")

plt.xlabel("Time (ms)")

plt.ylabel("A(t)")

plt.grid(True, alpha=0.3)

plt.show()

print(f" 參數確認：{freq_in.value} Hz | 總時長：{t_s[-1]*1000:.2f} ms")

except Exception as e:

print(f" 錯誤: {e}")

def save_mod_file(b):

try:

t_s, v = generate_wave_data(wave_type.value, freq_in.value, pw_in.value, rep_in.value)

save_path = path_in.value

if not save_path.lower().endswith('.modulation'): save_path += ".modulation"

with open(save_path, 'w', encoding='utf-8') as f:

f.write("#t            A(t)\n")

for ts, val in zip(t_s, v):

f.write(f"{ts:.10f}\t{val:.6f}\n")

with out:

print(f" 導出成功！檔案：{save_path}")

except Exception as e:

with out: print(f" 導出失敗: {e}")

# 4. 介面連動

def on_type_change(change):

if change['new'] == 'sine':

pw_container.layout.display = 'none'

else:

pw_container.layout.display = 'block'

show_preview(None)

wave_type.observe(on_type_change, names='value')

btn_preview.on_click(show_preview)

btn_save.on_click(save_mod_file)

# 顯示 UI

ui = widgets.VBox([

widgets.HTML("<h2>調變波型產生器</h2>"),

wave_type, freq_in, pw_container, rep_in, path_in,

widgets.HBox([btn_preview, btn_save]),

out

])

display(ui)

show_preview(None)