import numpy as np

import s4l_v1.document as document

import s4l_v1.model as model

SIMULATION_NAME = "LF"  # 求解器名稱

RESULT_INDEX = -1  # -1 means latest result

FIELD_NAME = "SAR(x,y,z,f0)" # 場域名稱

FIELD_GROUP_NAME = "Overall Field"

MARKER_RADIUS = 0.02 # Unit mm

def get_field_data_from_simulation(simulation_name, field_name, field_group_name="Overall Field", result_index=-1):

simulation = document.AllSimulations[simulation_name]

number_of_results = simulation.NumberOfResults

if number_of_results <= 0:

raise RuntimeError("Simulation has no results: {}".format(simulation_name))

if result_index < 0:

result_index = number_of_results + result_index

if result_index < 0 or result_index >= number_of_results:

raise RuntimeError(

"Invalid result index {} for simulation '{}'. NumberOfResults = {}".format(

result_index, simulation_name, number_of_results

)

)

# Same pattern as Sim4Life's built-in "To notebook" export:

# simulation = document.AllSimulations["LF"]

# simulation_extractor = simulation.Results()

simulation_extractor = simulation.Results(result_index)

print("Using simulation:", simulation_name)

print("Using result index:", result_index)

print("Available result groups:", list(simulation_extractor.keys()))

if field_group_name not in simulation_extractor.keys():

raise RuntimeError(

"Field group not found: {}. Available groups: {}".format(

field_group_name, list(simulation_extractor.keys())

)

)

sensor_extractor = simulation_extractor[field_group_name]

if hasattr(sensor_extractor, "FrequencySettings"):

sensor_extractor.FrequencySettings.ExtractedFrequency = u"All"

sensor_extractor.UpdateAll()

available_fields = [output.Description for output in sensor_extractor.Outputs]

print("Available fields:", available_fields)

if field_name not in available_fields:

raise RuntimeError(

"Field not found: {}. Available fields in '{}': {}".format(

field_name, field_group_name, available_fields

)

)

output = sensor_extractor.Outputs[field_name]

output.Update()

field_data = output.Data

if field_data is None:

raise RuntimeError("Output data is None for field: {}".format(field_name))

return field_data

def field_values_to_numpy(field_data, snapshot_index=0):

"""Return field values as a NumPy array from Sim4Life FieldData."""

values = np.asarray(field_data.Field(snapshot_index))

if values.size == 0:

raise RuntimeError("Field data is empty.")

return values

def get_grid(field_data):

grid = field_data.Grid

if grid is None:

raise RuntimeError("FieldData has no Grid.")

return grid

def magnitude(values):

values = np.asarray(values)

# Vector field: if the last dimension looks like x/y/z components, use vector norm.

if values.ndim >= 2 and values.shape[-1] in (3, 6):

return np.linalg.norm(values[..., :3], axis=-1)

# Scalar or complex scalar field.

return np.abs(values)

def index_to_position(grid, flat_index, number_of_values):

number_of_points = getattr(grid, "NumberOfPoints", None)

number_of_cells = getattr(grid, "NumberOfCells", None)

if number_of_points == number_of_values and hasattr(grid, "GetPointTransformed"):

return grid.GetPointTransformed(flat_index)

if number_of_points == number_of_values and hasattr(grid, "GetPoint"):

return grid.GetPoint(flat_index)

if number_of_cells == number_of_values and hasattr(grid, "GetCellCenter"):

return grid.GetCellCenter(flat_index)

raise RuntimeError(

"Cannot map field index to a point or cell. "

"Check Grid.NumberOfPoints / NumberOfCells versus the number of field values."

)

def create_max_marker(position, max_value, field_name):

point = model.CreatePoint(position)

point.Name = "Max {} point: {:.6g}".format(field_name, max_value)

sphere = model.CreateSolidSphere(position, MARKER_RADIUS)

sphere.Name = "Max {} marker: {:.6g}".format(field_name, max_value)

return point, sphere

field_data = get_field_data_from_simulation(

SIMULATION_NAME,

FIELD_NAME,

FIELD_GROUP_NAME,

RESULT_INDEX,

)

values = field_values_to_numpy(field_data)

mag = magnitude(values)

flat_index = int(np.nanargmax(mag))

max_value = float(np.ravel(mag)[flat_index])

grid = get_grid(field_data)

position = index_to_position(grid, flat_index, np.ravel(mag).size)

point, sphere = create_max_marker(position, max_value, FIELD_NAME)

print("Max field:", FIELD_NAME)

print("Max value:", max_value)

print("Flat index:", flat_index)

print("Position:", position)

print("Created marker:", point.Name, sphere.Name)