from __future__ import annotations

import os

import s4l_v1.model as model

import s4l_v1.model.image as image

# Example:

# MRI_IMAGE_PATH = r"D:\CustomerCase\sub01_T1.nii.gz"

MRI_IMAGE_PATH = r"D:\DATA\12 t1_mpr_ AXC.nii.gz"

# Add one or more .vtk/.stl/.obj/.trk/.tck paths.

# Example:

# TRACT_PATHS = [

#     r"D:\CustomerCase\left_cst.trk",

#     r"D:\CustomerCase\right_cst.vtk",

# ]

TRACT_PATHS = [

r"D:\DATA\Fiberbundle\FiberLL.vtk",

r"D:\DATA\Fiberbundle\FiberRR.vtk"

]

MAX_STREAMLINES = 300

STREAMLINE_STRIDE = 1

MIN_STREAMLINE_POINTS = 2

STREAMLINE_NAME_PREFIX = "Tract"

TRACT_COLOR = (0.0, 0.85, 1.0)

SUPPORTED_MESH_EXTENSIONS = {".vtk", ".stl", ".obj"}

SUPPORTED_STREAMLINE_EXTENSIONS = {".trk", ".tck"}

def clean_paths(paths):

return [path for path in paths if path and str(path).strip()]

def ras_to_lps(point):

x, y, z = [float(value) for value in point]

return [-x, -y, z]

def vec3(values):

return model.Vec3(float(values[0]), float(values[1]), float(values[2]))

def set_tract_color(entity):

try:

entity.Color = model.Color(float(TRACT_COLOR[0]), float(TRACT_COLOR[1]), float(TRACT_COLOR[2]))

except Exception:

pass

def normalize_import_result(entities):

if isinstance(entities, (list, tuple)):

return list(entities)

return [entities]

def print_entity_bbox(label, entities):

try:

bbox = model.GetBoundingBox(entities)

print(label, "bbox:", bbox)

except Exception as exc:

print(label, "bbox unavailable:", repr(exc))

def import_reference_mri():

if not MRI_IMAGE_PATH:

print("MRI_IMAGE_PATH is empty; skipping MRI import.")

return None

if not os.path.exists(MRI_IMAGE_PATH):

raise FileNotFoundError("MRI_IMAGE_PATH does not exist: " + MRI_IMAGE_PATH)

mri = image.ImportImage(MRI_IMAGE_PATH)

try:

mri.Name = "Reference_MRI"

except Exception:

pass

print("Imported MRI:", MRI_IMAGE_PATH)

for attr in ("Name", "Shape", "Spacing", "Transform", "HasTransform", "RelativeTransform"):

try:

print("  {}: {}".format(attr, getattr(mri, attr)))

except Exception as exc:

print("  {}: <unavailable: {}>".format(attr, exc))

print_entity_bbox("MRI", [mri])

return mri

def import_mesh_tract(path):

print("Importing mesh tract:", path)

entities = normalize_import_result(model.Import(path))

base = os.path.splitext(os.path.basename(path))[0]

for index, entity in enumerate(entities, start=1):

try:

entity.Name = "{}_{}".format(base, index)

except Exception:

pass

set_tract_color(entity)

print("  imported entity count:", len(entities))

print("  coordinate handling: direct import; mesh is assumed to already be in MRI/world mm coordinates")

print_entity_bbox("  tract", entities)

return entities

def load_streamlines(path):

try:

import nibabel as nib

except ImportError:

raise RuntimeError(

"Reading .trk/.tck requires nibabel in the Sim4Life Python "

"environment. Install nibabel or export the tract as .vtk/.stl/.obj."

)

tractogram = nib.streamlines.load(path)

return tractogram.streamlines

def create_polyline(points, name):

converted = [vec3(ras_to_lps(point)) for point in points]

polyline = model.CreatePolyLine(converted)

try:

polyline.Name = name

except Exception:

pass

set_tract_color(polyline)

return polyline

def import_streamline_tract(path):

print("Importing streamline tract:", path)

streamlines = load_streamlines(path)

base = os.path.splitext(os.path.basename(path))[0]

entities = []

used = 0

for index, streamline in enumerate(streamlines):

if STREAMLINE_STRIDE > 1 and index % int(STREAMLINE_STRIDE) != 0:

continue

if len(streamline) < MIN_STREAMLINE_POINTS:

continue

name = "{}_{}_{}".format(STREAMLINE_NAME_PREFIX, base, used + 1)

entities.append(create_polyline(streamline, name))

used += 1

if MAX_STREAMLINES and used >= int(MAX_STREAMLINES):

break

print("  created streamline polylines:", len(entities))

print("  coordinate handling: nibabel world/RAS+ mm converted to patient LPS mm")

if MAX_STREAMLINES:

print("  MAX_STREAMLINES:", MAX_STREAMLINES)

print_entity_bbox("  streamlines", entities)

return entities

def import_tract(path):

if not os.path.exists(path):

raise FileNotFoundError("Tract path does not exist: " + path)

extension = os.path.splitext(path)[1].lower()

if extension in SUPPORTED_MESH_EXTENSIONS:

return import_mesh_tract(path)

if extension in SUPPORTED_STREAMLINE_EXTENSIONS:

return import_streamline_tract(path)

raise RuntimeError(

"Unsupported tract extension '{}'. Supported: .vtk, .stl, .obj, .trk, .tck".format(extension)

)

def main():

print("=== Sim4Life MRI + tract import ===")

import_reference_mri()

paths = clean_paths(TRACT_PATHS)

if not paths:

raise RuntimeError("TRACT_PATHS is empty. Add at least one tract file path.")

all_entities = []

for path in paths:

all_entities.extend(import_tract(path))

print("Done. Created/imported tract entity count:", len(all_entities))

if __name__ == "__main__":

main()