from pathlib import Path

import numpy as np

import s4l_v1.analysis.core as core

import s4l_v1.document as document

import s4l_v1.simulation.emlf as emlf

# ========================== 需要填寫 ==========================

# 1. 已完成一次求解的 LF Ohmic-QS 模擬名稱。

SOURCE_SIM_NAME = "LFtest"

# 2. 要套用六分量 tensor 的皮質 MaterialSettings 名稱。

#    只有一個皮質材料範例：("Cortex",)

#    多個皮質材料範例：("L1", "L2", "L3", "L4", "L5", "L6")

#    Air、電極或其他非皮質材料不要填入。

MATERIAL_NAMES = ("Muscle",)

# 3. 六分量提供方式：

#    "arrays" = 直接在下方放入六個陣列，程式自動產生 NPZ

#    "file"   = 已經有合規 NPZ，直接指定檔案

TENSOR_SOURCE_MODE = "arrays"  # "arrays" 或 "file"

# arrays 模式不需修改這一行。只有改用 file 模式時，才填入既有 NPZ 路徑，

# 例如：ENSOR_INPUT_FILE = Path(r"D:\CustomerData\cortex_tensor.npz")

TENSOR_INPUT_FILE = None

# arrays 模式使用。正式資料請填 Python list 或 NumPy 一維陣列；長度必須等於 grid cell 數，排列順序必須與 Sim4Life cell ID 完全一致，單位為 S/m。

TENSOR_COMPONENTS = {

"XX": 0.6,

"YY": 0.2,

"ZZ": 0.2,

"XY": 0.0,

"YZ": 0.0,

"ZX": 0.0,

}

# ====================================================

ORDER = ("XX", "YY", "ZZ", "XY", "YZ", "ZX")

NEW_SIM_NAME = SOURCE_SIM_NAME + "_Tensor"

DATA_FOLDER = Path.home() / "Documents" / "Sim4Life_Tensor"

GRID_TEMPLATE = DATA_FOLDER / "tensor_grid_template.npz"

safe_sim_name = "".join(

character if character.isalnum() or character in "-_." else "_"

for character in SOURCE_SIM_NAME

)

AUTO_TENSOR_FILE = DATA_FOLDER / f"{safe_sim_name}_tensor_input_auto.npz"

if not SOURCE_SIM_NAME or SOURCE_SIM_NAME.startswith("請填入"):

raise RuntimeError("請填寫 SOURCE_SIM_NAME")

if (not MATERIAL_NAMES

or any(not name or name.startswith("請填入") for name in MATERIAL_NAMES)):

raise RuntimeError("請填寫 MATERIAL_NAMES")

if TENSOR_SOURCE_MODE not in ("file", "arrays"):

raise RuntimeError("TENSOR_SOURCE_MODE 必須是 file 或 arrays")

if TENSOR_SOURCE_MODE == "file":

if TENSOR_INPUT_FILE is None:

raise RuntimeError("file 模式請填寫 TENSOR_INPUT_FILE")

TENSOR_INPUT_FILE = Path(TENSOR_INPUT_FILE)

if TENSOR_INPUT_FILE.suffix.lower() != ".npz":

raise RuntimeError("TENSOR_INPUT_FILE 目前只接受 .npz 格式")

else:

if set(TENSOR_COMPONENTS) != set(ORDER):

raise RuntimeError(f"TENSOR_COMPONENTS 必須包含 {ORDER}")

if any(TENSOR_COMPONENTS[label] is None for label in ORDER):

raise RuntimeError("arrays 模式請填寫 TENSOR_COMPONENTS 的六個陣列")

if len(set(MATERIAL_NAMES)) != len(MATERIAL_NAMES):

raise RuntimeError("MATERIAL_NAMES 不可有重複名稱")

try:

source_sim = document.AllSimulations[SOURCE_SIM_NAME]

except KeyError as exc:

available = [s.Name for s in document.AllSimulations]

raise RuntimeError(f"找不到模擬；目前可用名稱：{available}") from exc

if not isinstance(source_sim, emlf.ElectroQsOhmicSimulation):

raise RuntimeError(f"指定的模擬不是 LF Ohmic-QS：{type(source_sim).__name__}")

if not source_sim.HasResults():

raise RuntimeError("指定的模擬尚無完整結果；請先完成一次求解")

if NEW_SIM_NAME in [s.Name for s in document.AllSimulations]:

raise RuntimeError(f"新模擬名稱已存在：{NEW_SIM_NAME}")

materials = [s for s in source_sim.AllSettings if isinstance(s, emlf.MaterialSettings)]

available_materials = [s.Name for s in materials]

if len(available_materials) != len(set(available_materials)):

raise RuntimeError("來源模擬有重複的 MaterialSettings 名稱，無法安全選取")

for name in MATERIAL_NAMES:

if name not in available_materials:

raise RuntimeError(f"找不到材料 '{name}'；目前材料：{available_materials}")

try:

e_output = source_sim.Results()["Overall Field"]["EM E(x,y,z,f0)"]

e_output.Update()

grid = e_output.Data.Grid

except (KeyError, AttributeError) as exc:

raise RuntimeError("既有結果需要 Overall Field / EM E(x,y,z,f0) 才能讀取 grid") from exc

axes = tuple(np.asarray(axis, dtype=np.float64)

for axis in (grid.XAxis, grid.YAxis, grid.ZAxis))

grid_units = tuple(str(grid.GetUnit(i)) for i in range(3))

if len(set(grid_units)) != 1:

raise RuntimeError(f"三個 grid 軸的長度單位不同：{grid_units}")

for axis in axes:

if len(axis) < 3 or not np.all(np.isfinite(axis)) or not np.all(np.diff(axis) > 0):

raise RuntimeError("來源 grid 軸必須有限、遞增，而且每方向至少有 2 個 cell")

cell_count = int(grid.NumberOfCells)

cell_centres = np.empty((cell_count, 3), dtype=np.float64)

for cell_id in range(cell_count):

point = grid.GetCellCenter(cell_id)

cell_centres[cell_id] = (point[0], point[1], point[2])

template = {

"cell_centres_grid_units": cell_centres,

"x_axis": axes[0],

"y_axis": axes[1],

"z_axis": axes[2],

"grid_length_unit": np.asarray(grid_units[0]),

"component_order": np.asarray(ORDER),

"conductivity_unit": np.asarray("S/m"),

}

for label in ORDER:

template[label] = np.full(cell_count, np.nan, dtype=np.float64)

DATA_FOLDER.mkdir(parents=True, exist_ok=True)

if GRID_TEMPLATE.exists():

with np.load(GRID_TEMPLATE, allow_pickle=False) as previous:

if not set(template).issubset(previous.files):

raise RuntimeError("現有模板格式較舊；請將它改名後重新執行")

for key in ("x_axis", "y_axis", "z_axis", "cell_centres_grid_units"):

if not np.array_equal(previous[key], template[key]):

raise RuntimeError("現有模板來自不同 grid；請改名保存後重新執行")

print("保留現有 grid 模板：", GRID_TEMPLATE)

else:

np.savez_compressed(GRID_TEMPLATE, **template)

print("已建立 grid 模板：", GRID_TEMPLATE)

print("Grid 參考模板：", GRID_TEMPLATE)

if TENSOR_SOURCE_MODE == "file":

print("將從客戶指定的 tensor 檔讀取六分量：", TENSOR_INPUT_FILE)

else:

print("將從 TENSOR_COMPONENTS 讀取六個陣列並自動建立 NPZ：", AUTO_TENSOR_FILE)

print("請確認六個陣列採用 Sim4Life cell ID 順序，單位為 S/m。")

print("Grid cell 數量：", cell_count)

print("Grid 長度單位：", grid_units[0])

print("可用 MaterialSettings：", available_materials)

if NEW_SIM_NAME in [s.Name for s in document.AllSimulations]:

raise RuntimeError(f"新模擬已存在：{NEW_SIM_NAME}；請勿重複執行本格")

if TENSOR_SOURCE_MODE == "file":

if not TENSOR_INPUT_FILE.is_file():

raise FileNotFoundError(f"找不到六分量輸入檔：{TENSOR_INPUT_FILE}")

with np.load(TENSOR_INPUT_FILE, allow_pickle=False) as data:

required = {

"cell_centres_grid_units", "x_axis", "y_axis", "z_axis",

"grid_length_unit", "component_order", "conductivity_unit",

*ORDER,

}

missing = required - set(data.files)

if missing:

raise RuntimeError(f"輸入檔缺少欄位：{sorted(missing)}")

for key, reference in (

("x_axis", axes[0]),

("y_axis", axes[1]),

("z_axis", axes[2]),

("cell_centres_grid_units", cell_centres),

):

actual = np.asarray(data[key], dtype=np.float64)

if actual.shape != reference.shape or not np.all(np.isfinite(actual)):

raise RuntimeError(f"{key} 的形狀錯誤或含非有限值")

scale = max(1.0, float(np.max(np.abs(reference))))

if np.max(np.abs(actual - reference)) > 1e-9 * scale:

raise RuntimeError(f"{key} 與既有模擬結果的 grid 不一致")

if str(data["grid_length_unit"].item()) != grid_units[0]:

raise RuntimeError("輸入檔的 grid 長度單位不符")

if str(data["conductivity_unit"].item()) != "S/m":

raise RuntimeError("conductivity_unit 必須是 S/m")

if tuple(str(value) for value in data["component_order"]) != ORDER:

raise RuntimeError(f"六分量順序必須是 {ORDER}")

tensor = np.column_stack([

np.asarray(data[label], dtype=np.float64) for label in ORDER

])

else:

component_arrays = []

scalar_components = []

for label in ORDER:

values = np.asarray(TENSOR_COMPONENTS[label], dtype=np.float64)

if values.ndim == 0:

scalar_components.append(label)

values = np.full(cell_count, float(values), dtype=np.float64)

elif values.ndim != 1:

raise RuntimeError(f"{label} 必須是單一測試數字或一維逐 cell 陣列")

component_arrays.append(values)

tensor = np.column_stack(component_arrays)

if scalar_components:

print(

"警告：以下分量使用固定值並套用到全部 cell：",

scalar_components,

)

print("")

if tensor.shape != (cell_count, 6) or not np.all(np.isfinite(tensor)):

raise RuntimeError("XX/YY/ZZ/XY/YZ/ZX 必須各有 N 個有限數值")

minimum_eigenvalue = np.inf

for start in range(0, cell_count, 10000):

values = tensor[start:start + 10000]

matrices = np.zeros((len(values), 3, 3), dtype=np.float64)

matrices[:, 0, 0] = values[:, 0]

matrices[:, 1, 1] = values[:, 1]

matrices[:, 2, 2] = values[:, 2]

matrices[:, 0, 1] = matrices[:, 1, 0] = values[:, 3]

matrices[:, 1, 2] = matrices[:, 2, 1] = values[:, 4]

matrices[:, 2, 0] = matrices[:, 0, 2] = values[:, 5]

chunk_minimum = float(np.min(np.linalg.eigvalsh(matrices)))

minimum_eigenvalue = min(minimum_eigenvalue, chunk_minimum)

if chunk_minimum <= 0:

raise RuntimeError(

f"Tensor 必須在每個 cell 都是正定；發現最小 eigenvalue={chunk_minimum}"

)

if TENSOR_SOURCE_MODE == "arrays":

generated_npz = {

"cell_centres_grid_units": cell_centres,

"x_axis": axes[0],

"y_axis": axes[1],

"z_axis": axes[2],

"grid_length_unit": np.asarray(grid_units[0]),

"component_order": np.asarray(ORDER),

"conductivity_unit": np.asarray("S/m"),

}

for index, label in enumerate(ORDER):

generated_npz[label] = tensor[:, index]

np.savez_compressed(AUTO_TENSOR_FILE, **generated_npz)

print("已由六分量自動建立 NPZ：", AUTO_TENSOR_FILE)

field = core.DoubleFieldData()

field.NumberOfSnapshots = 1

field.NumberOfComponents = 6

field.Grid = grid

field.ValueLocation = core.eValueLocation.kCellCenter

quantity = core.Quantity()

quantity.Name = "Electric Conductivity Tensor"

quantity.Unit = core.Unit("S/m", core.Unit.UnitPrefixEnum.kNoPrefix)

field.Quantity = quantity

for index, label in enumerate(ORDER):

field.SetComponentLabel(index, label)

field.SetComponentUnit(index, quantity.Unit)

field.SetField(0, tensor)

if not field.Check():

raise RuntimeError("六分量 DoubleFieldData 檢查失敗")

producer = core.TrivialProducer()

producer.Name = "Cortical_Conductivity_Tensor_" + NEW_SIM_NAME

producer.SetDataObject(field)

document.AllAlgorithms.Add(producer)

producer.UpdateAttributes()

producer.Update()

new_sim = source_sim.Clone()

new_sim.Name = NEW_SIM_NAME

for material_name in MATERIAL_NAMES:

matches = [

setting for setting in new_sim.AllSettings

if isinstance(setting, emlf.MaterialSettings) and setting.Name == material_name

]

if len(matches) != 1:

raise RuntimeError(f"複製後找不到唯一的材料：{material_name}")

material = matches[0]

material.ElectricProps.ConductivityAnisotropic = True

material.ElectricProps.ConductivityInhomogeneous = True

material.SetInhomogeneousConductivity(producer.Outputs[0])

if not (material.ElectricProps.ConductivityAnisotropic

and material.ElectricProps.ConductivityInhomogeneous):

raise RuntimeError(f"皮質材料的 anisotropic/inhomogeneous 設定失敗：{material_name}")

document.AllSimulations.Add(new_sim)

document.Save()

print("六分量 tensor 已通過檢查並完成材料指派。")

print("最小 eigenvalue：", minimum_eigenvalue, "S/m")

print("新模擬：", NEW_SIM_NAME)